Lab Sheet 1: Implement DSSS, line coding in MATLAB or equiv. tool

Introduction to MATLAB:

MATLAB is an interactive system whose basic data element is an array that does not require dimensioning. It stands for Matrix Laboratory.It is a software package for high-performance mathematical computation, visualization, and programming environment. It provides an interactive environment with hundreds of built-in functions for technical computing, graphics, and animations. It provides vast library of mathematical functions for linear algebra, statistics, Fourier analysis, filtering, optimization, numerical integration and solving ordinary differential equations.

MATLAB provides built-in graphics for visualizing data and tools for creating custom plots. MATLAB's programming interface gives development tools for improving code quality maintainability and maximizing performance. It provides tools for building applications with custom graphical interfaces. It provides functions for integrating MATLAB based algorithms with external applications and languages such as C, Java, .NET and Microsoft Excel.MATLAB was written initially to implement a simple approach to matrix software developed by the LINPACK (Linear system package) and EISPACK (Eigen system package) projects. MATLAB is a modern programming language environment, and it has refined data structures, includes built-in editing and debugging tools, and supports object-oriented programming. MATLAB is Multi-paradigm. So, it can work with multiple types of programming approaches, such as Functional, Object-Oriented, and Visual.

Some of the basic commands used in MATLAB are listed below:

clc		Clears command window.

clear		Removes variables from memory.

exist		Checks for existence of file or variable.

global		Declares variables to be global.

help		Searches for a help topic.

lookfor		Searches help entries for a keyword.

quit		Stops MATLAB.

who		Lists current variables.

cd		Changes current directory.

date		Displays current date.

delete		Deletes a file.

disp		Displays contents of an array or string.

fscanf		Read formatted data from a file.

fprintf		Performs formatted writes to screen or file.

input		Displays prompts and waits for input.

axis		Sets axis limits.

fplot		Intelligent plotting of functions.

grid		Displays gridlines.

plot		Generates xy plot.

print		Prints plot or saves plot to a file.

title		Puts text at top of plot.

xlabel		Adds text label to x-axis.

ylabel		Adds text label to y-axis.

cat		Concatenates arrays.

find		Finds indices of nonzero elements.

length		Computes number of elements.

max		Returns largest element.

min		Returns smallest element.

size		Computes array size.

sort		Sorts each column.

sum		Sums each column.

%s		Format as a string.

%d		Format as an integer.

%f		Format as a floating point value.

%e		Format as a floating point value in scientific notation.

%g		Format in the most compact form: %f or %e.

\n		Insert a new line in the output string.

\t		Insert a tab in the output string.

;		Suppresses screen printing.

Direct Sequence Spread Spectrum (DSSS)

Theory:

The Direct Sequence Spread Spectrum (DSSS) is a spread-spectrum modulation technique primarily used to reduce overall signal interference in telecommunication. The Direct Sequence Spread Spectrum modulation makes the transmitted signal wider in bandwidth than the information bandwidth. In DSSS, the message bits are modulated by a bit sequencing process known as a spreading sequence. This spreading-sequence bit is known as a chip. It has a much shorter duration (larger bandwidth) than the original message bits. DSSS increases transmit signal bandwidth to remove intersymbol interference (ISI) and narrowband interference. It provides security of transmission because receiver must know pseudo-random sequence to get transmitted data signal. Otherwise, receiver can’t detect original message signal, and it only sees transmitted signal as a noise. This property of DSSS technique make it suitable to use for military communication systems. DSSS also enables to usage of a bandwidth by multiple users because each user multiply its message signal by different pseudo-random sequence, and a receiver gets its message signal if it has transmitter pseudo-random sequence. The process of DSSS is shown below.

Figure : Block diagram of Spread Spectrum System

Code Implementation:

clear all;

%% parameters

Fs = 1000;

fc = 100;

fp = 4;

bit_t = 0.1;

%% message generation with BPSK

m = [0 0 1 1 1 1 0 0];

for bit = 1:length(m)

if(m(bit)==0)

m(bit) = -1;

end

end

message =  repmat(m,fp,1);

message =  reshape(message,1,[]);

%% PN generation and multiply with message

pn_code = randi([0,1],1,length(m)*fp);

for bit = 1:length(pn_code)

if(pn_code(bit)==0)

pn_code(bit) = -1;

end

end

DSSS = message.*pn_code;

%% create carrier and multipy with encoded sequence

t = 0:1/Fs:(bit_t-1/Fs);

s0 = -1*cos(2*pi*fc*t);

s1 = cos(2*pi*fc*t);

carrier = [];

BPSK = [];

for i = 1:length(DSSS)

if (DSSS(i) == 1)

BPSK = [BPSK s1];

elseif (DSSS(i) == -1)

BPSK = [BPSK s0];

end

carrier = [carrier s1];

end

%% demodulation

rx =[];

for i = 1:length(pn_code)

if(pn_code(i)==1)

rx = [rx BPSK((((i-1)*length(t))+1):i*length(t))];

else

rx = [rx (-1)*BPSK((((i-1)*length(t))+1):i*length(t))];

end

end

demod = rx.*carrier;

result = [];

for i = 1:length(m)

x = length(t)*fp;

cx = sum(carrier(((i-1)*x)+1:i*x).*demod(((i-1)*x)+1:i*x));

if(cx>0)

result = [result 1];

else

result = [result -1];

end

end

pn_codeWrong = randi([0,1],1,length(m)*fp);

resultWrong = [];

rx2 =[];

for i = 1:length(pn_code)

if(pn_codeWrong(i)==1)

rx2 = [rx2 BPSK((((i-1)*length(t))+1):i*length(t))];

else

rx2 = [rx2 (-1)*BPSK((((i-1)*length(t))+1):i*length(t))];

end

end

demod2 = rx2.*carrier;

for i = 1:length(m)

x = length(t)*fp;

cx = sum(carrier(((i-1)*x)+1:i*x).*demod2(((i-1)*x)+1:i*x));

if(cx>0)

resultWrong = [resultWrong 1];

else

resultWrong = [resultWrong -1];

end

end

message1 =  repmat(result,fp,1);

message1 =  reshape(message1,1,[]);

message2 =  repmat(resultWrong,fp,1);

message2 =  reshape(message2,1,[]);

%% Draw original message, PN code , encoded sequence on time domain

pn_size = length(pn_code);

tpn = linspace(0,length(m)*bit_t-bit_t/fp,pn_size);

tm = 0:bit_t/fp:length(m)*bit_t-bit_t/fp;

figure

subplot(311)

stairs(tm,message,'linewidth',2)

title('Message bit sequence')

axis([0 length(m)*bit_t -1 1]);

subplot(312)

stairs(tpn,pn_code,'linewidth',2)

title('Pseudo-random code');

axis([0 length(m)*bit_t -1 1]);

subplot(313)

stairs(tpn,DSSS,'linewidth',2)

title('Modulated signal');

axis([0 length(m)*bit_t -1 1]);

figure

subplot(311)

stairs(tm,message,'linewidth',2)

title('Message bit sequence')

axis([0 length(m)*bit_t -1 1]);

subplot(312)

stairs(tm,message1,'linewidth',2)

title('Received message using true pseudo-random code')

axis([0 length(m)*bit_t -1 1]);

subplot(313)

stairs(tm,message2,'linewidth',2)

title('Received message using wrong pseudo-random code')

axis([0 length(m)*bit_t -1 1]);

%% Draw original message, PN code , encoded sequence on frequency domain

f = linspace(-Fs/2,Fs/2,1024);

figure

subplot(311)

plot(f,abs(fftshift(fft(message,1024))),'linewidth',2);

title('Message spectrum')

subplot(312)

plot(f,abs(fftshift(fft(pn_code,1024))),'linewidth',2);

title('Pseudo-random code spectrum');

subplot(313)

plot(f,abs(fftshift(fft(DSSS,1024))),'linewidth',2);

title('Modulated signal spectrum');

figure;

subplot(311)

plot(f,abs(fftshift(fft(BPSK,1024))),'linewidth',2);

title('Transmitted signal spectrum');

subplot(312)

plot(f,abs(fftshift(fft(rx,1024))),'linewidth',2);

title('Received signal multiplied by pseudo code');

subplot(313)

plot(f,abs(fftshift(fft(demod,1024))),'linewidth',2);

title('Demodulated signal spectrum before decision device ');

Output:                     Figure 1: Message bit sequence, Pseudo random code and Modulated signal

Figure 2: Original Message and results with true and wrong key at receiver

Figure3: Message spectrum, Pseudo random code spectrum and Modulated signal spectrum

Figure 4: Draw original message, PN code , encoded sequence on frequency domain

Line Coding

Theory:

The line coding is defined as the process of converting binary data,a sequence of bits to a digital signal.The digital data such as text, numbers,graphical images,audio and video are stored in computer memory in the form of sequences of bits. Data as well as signals that represents data can either be digital or analog. Line coding is the process of converting digital data to digital signals. Whenever we transmit data it is in the form of digital signals, so with the help of line coding, we can convert a sequence to bits (or encoding) into a digital signal which then again converted into bits by the receiver (or can be said as decoded by the receiver). A line code is the code used for data transmission of a digital signal over a transmission line. This process of coding is chosen so as to avoid overlap and distortion of signal such as inter-symbol interference.

Code Implementation:

N=10

n=randi([0,1],1,N)

for m=1:N

if n(m)==1

nn(m)=1;

else

nn(m)=0;

endif

endfor

i=1

t=0:0.01:length(n);

for j=1:length(t)

if t(j)<=i

y(j)=nn(i);

else

i=i+1

y(j)=nn(i);

endif

endfor

plot(t,y);

axis([1,N,-2,2])

Output:

Figure: Line coding

Lab Sheet 2: Analyze the performance of  WiFi network using NetSim or equiv. tool.

Introduction to Netsim (Network Simulator)

NetSim is an end-to-end, full stack, packet level network simulator and emulator. It provides network engineers with a technology development environment for protocol modeling, network R&D and military communications. The behavior and performance of new protocols and devices can be investigated in an virtual network within NetSim at significantly lower cost and in less time than with hardware prototypes.It is software that predicts the behavior of a computer network. Since communication networks have become too complex for traditional analytical methods to provide an accurate understanding of system behavior, network simulators are used. In simulators, the computer network is modeled with devices,links, applications etc. and the network performance is reported. Simulators come with support for the most popular technologies and networks in use today such as 5G, Internet of Things (IoT), Wireless LANs, mobile ad hoc networks, wireless sensor networks, vehicular ad hoc networks, cognitive radio networks, LTE etc.

WiFi(Wireless Fidelity)

Theory:

Wi-Fi is a family of wireless network protocols, based on the IEEE 802.11 family of standards, which are commonly used for local area networking of devices and Internet access. The WiFi standards define a fixed channel bandwidth of 25 MHz for 802.11b and 20 MHz for either 802.11a or g networks. primary channel access mechanism is CSMA/CA Wi‑Fi is a trademark of the non-profit Wi-Fi Alliance, which restricts the use of the term Wi-Fi Certified to products that successfully complete interoperability certification testing.

Wi-Fi uses multiple parts of the IEEE 802 protocol family and is designed to interwork seamlessly with its wired sibling Ethernet. Compatible devices can network through wireless access points to each other as well as to wired devices and the Internet. The different versions of Wi-Fi are specified by various IEEE 802.11 protocol standards, with the different radio technologies determining radio bands, and the maximum ranges, and speeds that may be achieved. Wi-Fi most commonly uses the 2.4 gigahertz (120 mm) UHF and 5 gigahertz (60 mm) SHF ISM radio bands; these bands are subdivided into multiple channels. Channels can be shared between networks but only one transmitter can locally transmit on a channel at any moment in time.Performance of a WiFi link and system depends on signal quality, interference and so on.

Signal Quality:

Signal qualtity is the function of transmitted power, path loss, noise, shadowing, etc. Poor signal quality leads to packet errors and losses. Quality is how much interference there is between the satellite and your decoder. Signal quality is how good the information you are recieving actually is All signals hold reflections/ghosting or interference that we call noise. Noise will cause the loss of data. The noise will be low if the quality is high or good. If the the signal is suffering due to reflections or interfernce from electrical installations switches/motors etc then the quality will be poor as these problems will cause the loss of data in the signal as the effect of them will block/overpower the signal and your box won't be able to "read" the true data. Without any of these problems the your quality will be high.

Interference:

Interference is that which modifies a signal in a disruptive manner, as it travels along a communication channel between its source and receiver. The term is often used to refer to the addition of unwanted signals to a useful signal. Interference occurs when unwanted signals disrupt wireless communication.Interference may prevent reception altogether, may cause only a temporary loss of a signal, or may affect the quality of the audio or video produced by your equipment. Interference is the primitive nature of the wireless communication system. It is the case in which multiple transmission occurs many times simultaneously over a common communication medium. Interference can come from WiFi and non WiFi radios waves.It leads to poor channel utilization, packet errors and losses.

Implementation:

Network Setup: An IEEE 802.11n link between an access point and a client downlink UDP traffic between a server and the client was configured as shown in figure below:

Figure: IEEE 802.11n network topology

| MCS | Modulation Type | Coding Rate | Data Rate |
| --- | --- | --- | --- |
| 0 | BPSK | 1/2 | 7.2 |
| 1 | QPSK | 1/2 | 14.4 |

IEEE 802.11n supports a fixed set of data rates among which we had used BPSK and QPSK both having coding rate of ½.

Table: Modulation type and it’s Data rate

Rate Adaptation: Experiment Configuration

| WiFi Radio |  |
| --- | --- |
| Standard | 802.11n |
| Band | 2.4 GHz |
| Channel | 1 |
| Bandwidth | 20 MHz |
| Transmit Power | 20 dBm and 10 dBm |
| Rate Adaptation | False |

| Wireless Channel and Link |  |
| --- | --- |
| Channel Characteristics | Path Loss Only |
| Path Loss Model | Log Distance |
| Path Loss Exponent | 3.5 |
| Mobility Model | Constant |
| AP - Client Distance | Variable |

| Application Properties |  |
| --- | --- |
| Application | CBR |
| Transport Protocol | UDP |
| Packet Size | 1450 bytes |
| Interarrival Time | 116 microseconds |
| TCP | Disabled |

| Miscellaneous |  |
| --- | --- |
| Wired Link Capacity Wired Link Delay Simulation Time | 1 Gbps 10 microseconds 10 seconds |

Performance Measure:

- Data rate (from Packet Trace):

DATARATE=      			PHY LAYER PAYLOAD∗8

(PHY LAYER END TIME −PHY LAYER ARRIVAL TIME −40)

- Average throughput (from Packet Trace):

AVERAGE THROUGHPUT = 	 APPLICATION BYTES RECEIVED ∗8.0

SIMULATION TIME

Output:

Figure: Relation between Data rate and throughput with distance

Figure: Relation between data rate with distance and transmit power

Conclusion:

Thus, data rate and throughput decreases with distance. Also data rate decreases as transmit power decreases and coverage (range) decreases as transmit power decreases.

Lab Sheet 3:

Develop QPSK detector and understand the relation between BER and SNR.

Theory:

Modulation is the process of varying one or more properties of a periodic waveform, called the carrier signal, with a separate signal called the modulation signal that typically contains information to be transmitted Modulation is the process of changing the parameters of the carrier signal, in accordance with the instantaneous values of the modulating signal. The purpose of modulation is to impress the information on the carrier wave, which is used to carry the information to another location. The baseband signals are incompatible for direct transmission. For such a signal, to travel longer distances, its strength has to be increased by modulating with a high frequency carrier wave, which doesn’t affect the parameters of the modulating signal.

In digital modulation, an analog carrier signal is modulated by a discrete signal. Digital modulation methods can be considered as digital-to-analog conversion and the corresponding demodulation or detection as analog-to-digital conversion. The changes in the carrier signal are chosen from a finite number of M alternative symbols (the modulation alphabet). Digital Modulation provides more information capacity, high data security, quicker system availability with great quality communication. Hence, digital modulation techniques have a greater demand, for their capacity to convey larger amounts of data than analog modulation techniques.

SNR: Signal-to-noise ratio (SNR or S/N) is a measure used in science and engineering that compares the level of a desired signal to the level of background noise. SNR is defined as the ratio of signal power to the noise power, often expressed in decibels. Signal-to-noise ratio (SNR or S/N) is a measure used in science and engineering that compares the level of a desired signal to the level of background noise. SNR is defined as the ratio of signal power to the noise power, often expressed in decibels.

If Vs = Vn, then S/N = 0. In this situation, the signal borders on unreadable, because the noise level severely competes with it. In digital communications, this will probably cause a reduction in data speed because of frequent errors that require the source (transmitting) computer or terminal to resend some packets of data.

A ratio higher than 1:1 (greater than 0 dB) indicates more signal than noise.

BER: When data is transmitted over a data link, there is a possibility of errors being introduced into the system. If errors are introduced into the data, then the integrity of the system may be compromised. As a result, it is necessary to assess the performance of the system, and bit error rate, BER, provides an ideal way in which this can be achieved. Unlike many other forms of assessment, bit error rate, BER assesses the full end to end performance of a system including the transmitter, receiver and the medium between the two. In this way, bit error rate, BER enables the actual performance of a system in operation to be tested, rather than testing the component parts and hoping that they will operate satisfactorily when in place.

the number of bit errors is the number of received bits of a data stream over a communication channel that have been altered due to noise, interference, distortion or bit synchronization errors. The bit error rate (BER) is the number of bit errors per unit time. a bit error rate is defined as the rate at which errors occur in a transmission system. This can be directly translated into the number of errors that occur in a string of a stated number of bits. The definition of bit error rate can be translated into a simple formula:

BER=Errors/Total Number of Bits

If the medium between the transmitter and receiver is good and the signal to noise ratio is high, then the bit error rate will be very small - possibly insignificant and having no noticeable effect on the overall system However if noise can be detected, then there is chance that the bit error rate will need to be considered.

Implementation:

https://www.diva-portal.org/smash/get/diva2:829374/FULLTEXT01.pdf for files

Lab Sheet 4:

Implement various pulse shaping filters implemented in wireless communication.

Theory:

Pulse shaping plays a crucial role in spectral shaping in the modern wireless communication to reduce the spectral bandwidth. Pulse shaping is a spectral processing technique by which fractional out of band power is reduced for low cost, reliable , power and spectrally efficient mobile radio communication systems. pulse shaping is the process of changing the waveform of transmitted pulses. Its purpose is to make the transmitted signal better suited to its purpose or the communication channel, typically by limiting the effective bandwidth of the transmission. By filtering the transmitted pulses this way, the inter symbol interference caused by the channel can be kept in control. In RF communication, pulse shaping is essential for making the signal fit in its frequency band. It is clear that the pulse shaping filter not only reduces inter-symbol interference (ISI), but it also reduces adjacent channel interference. Typically pulse shaping occurs after line coding and modulation.

Rectangular Pulse: The most basic information unit in a digital transmission scheme is a rectangular pulse. It has a defined amplitude, A, and defined duration, T. Such a pulse is shown in Figure 7, where A = 1, T = To, with the pulse centered about the time origin at t = 0. Typically, a sequence of such pulses (each delayed by T seconds relative to the previous one) constitutes the transmission of information. The information, in this case, is encoded in the amplitude of the pulse. The simplest case is when a binary 0 is encoded as the absence of a pulse (A = 0) and a binary 1 is encoded as the presence of a pulse (A = constant). Since each pulse spans the period T, the maximum pulse rate is 1/T pulses per second, which leads to a data transmission rate of 1/T bits per second.

The pulses used to transmit symbols occupy a fixed time interval, T .Thus, the pulse rate is 1/T pulses per second, which leads to a symbol rate of 1/T symbols per second. The unit, symbols per second, is often referred to as baud. The data transmission rate in bits per second is the baud rate multiplied by the number of bits represented by each symbol. For example, if a symbol represents four bits, then the bit rate is four times the symbol rate. This means that a lower transmission rate can be used to transmit symbols as opposed to directly transmitting bits, which is the primary reason that the more sophisticated data transmission systems encode groups of bits into symbols. The logic 1 is represented by the presence of a pulse of unit amplitude and a logic 0 by the absence of a pulse (that is, zero amplitude).

The rectangular pulse is defined as

where A is the amplitude of the pulse and L is an integer. The frequency domain representation of the rectangular pulse is

The Fourier transform of the rectangular pulse is real and its spectrum, a sinc function, is unbounded. This is equivalent to an upsampled pulse-train of upsampling factor L. In real systems, rectangular pulses are spectrally bounded via filtering before transmission which results in pulses with finite rise and decay time.

Implementation:

Lab Sheet 5:

Implement wireless routing protocols: DSDV & AODV

Theory:

ADOV: The Ad hoc On Demand Distance Vector (AODV) routing algorithm is a routing protocol designed for ad hoc mobile networks. AODV is capable of both unicast and multicast routing. It is an on-demand algorithm, meaning that it builds routes between nodes only as desired by source nodes. It maintains these routes as long as they are needed by the source. Ad hoc On-demand Distance Vector routing (AODV) protocol enables dynamic, self-starting, multi-hop routing between mobile nodes to establish the ad hoc network. The mobile nodes obtain routes only for those destinations that are in the active communication. Link breakages are detected by the affected set of nodes and they invalidate the routes using the lost link. When a source node desires to establish a communication session, it initiates a path-discovery process to locate the other node. AODV builds routes using route requested and route reply mechanisms. In order to discover the path, a route request (RREQ) packet is broadcasted across the network to find the route to the destination. Nodes receiving this packet update their information for the source node and set up backwards pointers to the source node in the route tables. To find a path to the destination, the source a initiates Route Request (RREQ) packet across the network and it contains the source address, destination address, source sequence number, destination sequence number, the broadcast identifier and the time to live field. Nodes keep track of the RREQ's source IP address and broadcast ID. If they receive a RREQ which they have already processed, they discard the RREQ and do not forward it. When a node forwards a RREQ packet to its neighbors, it also records in its routing table the node from which the first copy came and it is required by the node to construct the reverse path for the RREP packet. Information about the preceding node from which the packet was received is recorded when a node receives a RREP packet, in turn to forward the data packets to this next node as the next hop toward the destination. Once the source node receives a RREP it can begin using the route to send data packets. Figure 1 shows the propagation of the RREQ across the network.

Figure 1. Route Request flooding

Once the RREQ reaches the destination or an intermediate node with a fresh enough route, this node responds by unicasting a route reply (RREP) packet back to the neighbor from which it received the RREQ packet. The path which follows the RREP message is shown in Figure 2.

Figure 2. Route Reply propagation

As long as the route remains active, it will continue to be maintained. A route is considered active as long as there are data packets periodically traveling from the source to the destination along that path. Once the source stops sending data packets, the links will time out and eventually be deleted from the intermediate node routing tables. If a link break occurs while the route is active, the node upstream of the break propagates a route error (RERR) message to the source node to inform it of the now unreachable destination(s). After receiving the RERR, if the source node still desires the route, it can reinitiate route discovery process.

DSDV: DSDV (Destination Sequenced Distance Vector) is a table-driven algorithm based on the classical Bellman-Ford routing mechanism, but guaranteeing loop-freedom via sequence numbers. Every mobile node in the network maintains a routing table in which all of possible destination within the network and the number of hops to each destination are recorded. Each entry is marked with a sequence number assigned by the destination node. The sequence numbers enable the mobile nodes to distinguish stale routes from newer ones, avoiding the formation of routing loops. Update messages contain the address of the destination, the number of hops to reach the destination and the sequence number of the advertised route. Routes labelled with the most recent sequence number are always preferred. In the event that two updates have the same sequence number, the route with the smaller metric is used. Routes availability to all destinations implies that much less delay is involved in route setup process. The data broadcast by each node will contain its new sequence number, the destination’s address, the number of hops count.

Figure:  DSDV routing table for above nodes

Implementation:

Network Topology: The following diagram shows a scenario with a topology of 100 nodes. Node 0 is the source which transmits data. Node 100 is the sink or the destination for the whole network.

Figure. A WSN Topology with 100 nodes used in simulation

PERFORMANCE METRICS: Various Quality of Service parameters used for analysis routing protocols are defined as follows.

Packet Delivery Ratio (PDR): It is the ratio of deliver packet which is send by the source node and received by the destination node. When packet delivery ratio is high then performance is better . Mathematically, it can be written as in this equation:

PDR is calculated in % (percentage). Higher values of PDR carry better performance.

Average throughput: It is the ratio between the actual numbers of packets transmitted by the nodes in the system to the number of successfully delivered packets at the base station. The throughput is usually measured in bits per second (bit/sec), and sometimes in data packets per second or data packets per time slot. Higher throughput is always desirable in a communication system. The average throughput is given as follows:

Where:

recdSize = Store received packet ‘s size

Stop Time = Simulation stop time

startTime = Simulation start time

End to End delay: End-to-end delay refers to the time taken for a packet to be transmitted across a network from source to destination. A data packet may take longer time to reach to the destination due to queuing and different routing paths. It is derived in ms (mille second). Smaller values of End-to-end delay carries improved performance. The End-to-end delay is described as:

Packet Loss Ratio: Packet loss ratio is the number of packets that never reached the destination to the number of packets originated by the source. We aim to decrease the packet loss ratio. The packet loss ratio is given as:

Simulation Results:

In this section, the performance analysis is carried out on DSDV as proactive candidate and AODV as reactive representative, Performance metrics like Throughput, Packet Delivery Ratio, Dropped Packet, End to End Delay are the four common measures used for the comparison of the performance of above protocols. In the  scenario, the density of nodes varies from 10 to 100 nodes. In order to enable direct fair comparisons between the protocols, it was critical to challenge the protocols with identical loads and environmental conditions. Each run of the simulator accepts an input scenario file which describes the exact motion of each node. Since the  chosen protocols were challenged with the same scenario file and during the same time (500 seconds), we can directly compare the performance results of all protocols carried out. Simulations were performed by using Network simulator 2 (NS 2.35) Performance metrics are calculated from trace file, with the help of AWK program and it is plotted with the help of Microsoft Excel 2007 tool. The analysis result helps the network designer to choose right protocol. Simulation results are shown in the following section in form of line graphs.

In this scenario, number of nodes connected in a network at a time is varied and thus varying the number of connections, through which the comparison graphs of AODV and DSDV protocols, is made. All nodes are fixed at one place. Table 1 shows the main characteristics used for scenario

Table 1: Various parameters for scenario

| Parameter | Value |
| --- | --- |
| Routing Protocols | AODV, DSDV and ZRP |
| Number of nodes | 10, 25, 50,7 5 and100 |
| Simulation Time | 500 seconds |
| Traffic Type/Network Protocol | CBR/UDP |
| Bandwidth | 0.4 Mb |
| Packet size | 1500 bytes |
| Mobility Model | Off: A Fixed Topology |
| Radio Propagation Model | Two Ray Ground |
| Channel Type | Wireless channel |
| Queue files | Queue/Drop Tail/Prique |
| Queue length | 50 |
| Mac layer | 802.11 |
| Antenna Type | Omni Antenna |
| Topology size | 1200 x 1200 |

Table 2 and table 3 give performances results of the routing protocols, with varying the number of nodes from within a margin of 10 to 100 nodes. Network traffic type is chosen as CBR (Constant Bit Rate). The routing protocols are set as AODV and DSDV to compare the simulation data. The performance metrics used for comparison are Average Throughput, End- to-End delay, Packet Loss Ratio, and Delivered Packet Ratio.

Table 2. DSDV Evaluation for scenario 1

| Number of Nodes | Number of Packet Loss | Delivered Packet Ratio | Packet Loss Ratio | Average throughput | End to End Delay |
| --- | --- | --- | --- | --- | --- |
| 10 | 1456 | 90.78 | 9.22 | 144.45 | 309.51 |
| 25 | 692 | 93.90 | 6.09 | 100.57 | 147.84 |
| 50 | 988 | 91.55 | 8.44 | 104.53 | 231.83 |
| 75 | 599 | 90.91 | 9.08 | 58 | 166.27 |
| 100 | 461 | 91.27 | 8.73 | 46 | 171.47 |

Table 3. AODV Evaluation for scenario

| Number of Nodes | Number of Packet Loss | Delivered Packet Ratio | Packet Loss Ratio | Average throughput | End to End Delay |
| --- | --- | --- | --- | --- | --- |
| 10 | 4701 | 77.85 | 22.15 | 182.48 | 301.27 |
| 25 | 1595 | 90.59 | 6.09 | 142.8 | 160.78 |
| 50 | 1881 | 86.91 | 8.44 | 120.68 | 138.84 |
| 75 | 1234 | 89.6 | 11.08 | 98.89 | 156.76 |
| 100 | 1175 | 88.92 | 8.73 | 46 | 157.31 |

All results are analyzed and briefed in form of graphs given below. These graphs are found very helpful in statistical analysis of these routing protocols. The required graphs were saved as the bitmap image for statistical analysis. In this figure, we estimate the average throughput for all three routing protocols namely AODV and DSDV

Figure A. Packet Delivery Ratio for DSDV, AODV

In Figure A we see the performance illustration of Packet Delivery Ratio depending on the number of nodes, we notice that the value of Packet Delivery Ratio in DSDV remains constant and AODV shows variation. AODV performance dropped as number of nodes increase because more packets dropped due to link breaks. When we analyse where these lost packets are in AODV, we notice that AODV has not only more packets in buffers waiting for a route; but also, more packets are lost because they were sent following old routes. So AODV, suffers in part from its lack of periodic update information but maintaining reasonably good delivery ratio.